Question 1: What is the purpose of the loopback interface on the Cisco router?

Answer: It is a logical (virtual) interface that cannot physically breakdown, and is usually used as a router ID in router protocols such as OSPF, and for testing.

Answer: It's a logical (virtual) interface that cannot fail physically and is commonly used as a Router ID in routing protocols like OSPF, as well as for testing.

*Question 2: What is a wildcard mask, and what is it used for?

Answer: It is an inverse mask used in Access Control Lists (ACLs) and orientation protocols to determine a range of addresses rather than a single address.

Answer: It's an inverse mask used in Access Control Lists (ACLs) and routing protocols to specify a range of addresses instead of a single one.

*Question 3: In OSPF protocol, what is the Stub network?

Answer: Is an area with no external routes, used to reduce the size of the orientation database by not receiving external routes updates.

Answer: It's an area that contains no external routes and is used to reduce the size of the routing database by not receiving external route updates.

*Question 4: In IPv6 titles, what is the main purpose of operation EUI-64?

Answer: To automatically create a globally unique IPv6 interface (Interface ID) using the interface's MAC address.

Answer: To automatically create a globally unique IPv6 Interface ID using the interface's MAC address.

*Question 5: What are the three main types of memory in the Cisco router?

Answer: RAM, NVRAM, and Flash Memory.

Answer: RAM (Random Access Memory), NVRAM (Non-Volatile RAM), and Flash Memory.

Day 38 – What is a Proxy Server in Networking?

Welcome back to our Learning Networking series! Today, we'll address an important concept that is widely used in networks for security, privacy and performance optimization purposes:

Proxy Server (Proxy Server)

What is a proxy server?

The proxy server, or proxy, is a server that acts as an intermediary between your device (the client) and the Internet.

Instead of your device calling directly to the website or service you want, it first sends the request to the agent server. The agent server then redirects the request on your behalf to its destination, then receives the response and sends it to you.

Imagine wanting to order a meal from a restaurant.

*Without Broxy: Calls the restaurant directly and places your order.

*with Proxy: You call your "personal assistant" (Proxy), and give him your request.

The server calls the restaurant and places the order in his name, then picks up the meal and returns to give it to you.

Because of this process, the restaurant (website) sees that the order came from the assistant (proxy) and not you directly.

- The agent server is an intermediary located between the client and the Internet, and handles user requests on their behalf.

- Why do we use proxy servers (proxy servers)?

Agent servers offer many important benefits:

*privacy and anonymity:

*Representative server hides the real IP address of your device.

When you connect to a website through Proxy, the website only sees the agent server IP address, increasing your privacy.

* Performance Improving (Caching):

*The agent server can store pages and sites frequently visited by users.

* When the last user requests the same page, the agent server does not need to connect to the Internet again, it sends his stored version, which increases the download speed.

* Security and Control :

*A proxy server can act as a simple firewall.

Network administrators can use it to block malicious or inappropriate sites, preventing employees from accessing them.

*Can also be used to check traffic to look for viruses or malware before it reaches your device.

*the geographical restrictions exceeded:

* If there is a blocked service in your country, you can use an agent server located in another unblocked country, which allows you to access that service.

- Agent servers improve privacy, increase access speed, and provide an extra layer of security and control.

Types of agent servers

There are different types of agent servers, each with its own function:

*Forward Proxy):

*This is the most common type. Used to protect the internal network of an institution or individuals. It's between the user and the Internet, and guides external orders.

* Reverse Proxy (Reverse Proxy)

* This type of agents are in front of the organization's web server (not the user).

* Its function is to protect the web server, distribute traffic on several servers (Load Balancing), and provide security services.

The front agent protects the user, while the reverse agent protects the server.

Understanding agent servers is an essential part of understanding how modern networks work, especially in cybersecurity.

Have you ever used broxy? And what purpose did you use it for? Share with us in the comments!

Here are the top 20 used ports:

FTP – TCP 21 → To transfer files.

SSH – TCP 22 → to register safe entry to servers.

Telnet – TCP 23 → Remote login (unsafe).

SMTP – TCP 25 → Send email.

DNS – TCP/UDP 53 → Domain name queries.

DHCP Server – UDP 67 → Auto-distribute IP addresses.

DHCP Client – UDP 68 → Request IP address from server.

HTTP – TCP 80 → Browse websites (unencrypted).

POP3 – TCP 110 → Email Reception.

مز NTP – UDP 123 → Synchronizing time across the grid.

NetBIOS – TCP 139 → Network file sharing services.

IMAP – TCP 143 → Access Email.

HTTPS – TCP 443 → Secure Browsing (SSL/TLS).

SMB – TCP 445 → Sharing files and printers.

Oracle DB – TCP 1521 → Oracle Databases.

MySQL – TCP 3306 → MySQL Databases.

RDP – TCP 3389 → Remote Control (Remote Desktop).

PostgreSQL – TCP 5432 → PostgreSQl Databases.

LDAP – TCP/UDP 389 → Active Directory Services.

SNMP – UDP 161 → Network Devices Management.

What are the Access Control Lists (ACLs)?

Access control lists are a set of rules used to filter network data movement (Traffic Filtering), to determine who is allowed access to certain resources or services, and who is blocked. Commonly used in routers and firewalls.

What are Access Control Lists (ACLs)?

Access Control Lists are sets of rules used to filter network traffic, determining who is allowed to access specific resources or services and who is denied. They are commonly implemented in routers and firewalls.

Types of ACLs

1. Standard ACL - depends on source IP address only.

2. Extended ACL - Depends on source address and destination, as well as protocols and ports.

Types of ACLs

1. Standard ACL – Filters based only on the source IP address.

2. Extended ACL – Filters based on source and destination IP addresses, as well as protocols and ports.

The importance of ACLs

Strengthening the security of the network.

Control the flow of data.

Reducing the consumption of resources.

Importance of ACLs

Enhance network security.

Control data flow.

Reduce resource consumption.

Applied example (Cisco)

access-list 10 permit 192.168.1.0 0.0.0.255

access-list 10 deny any

This example allows any movement from the network 192.168.1.0 to pass and prevents the rest of the communications.

Example (Cisco)

This example allows traffic from the 192.168.1.0 network and denies all other connections.

Tip of the day:

Using ACLs correctly helps you protect your network from unauthorized access, significantly improving its performance.

Tasks of the IT department in companies and institutions - from behind the scenes to the heart of the work!

The IT department is the backbone of any modern enterprise. You may not always see them upfront, but they ensure every system, device, and network is running smoothly and safely.

1. Network Management

Design, installation and maintenance of network infrastructure.

Monitor network performance and ensure its continuity without interruption.

Solving communication issues between devices and systems.

2. User Support (Technical Support)

Receiving requests for technical support from staff and solving technical issues.

Training staff on software and tools.

Tackle emergency breakdowns quickly to minimize shutdowns.

3. Hardware Management

Installation and maintenance of computers, printers and servers.

Upgrade or replace devices as needed.

Maintain a clean and organized technical work environment.

4. Information security (Cybersecurity )

Protecting networks and systems from cyber attacks.

Management of Firewalls (IDS/IPS) and Intruder Detection Systems (Firewalls)

Implementing and constantly updating strict security policies.

5. Software Management (Software Management)

Installing and updating the necessary software for work.

Managing software licenses and ensuring legal compliance.

Developing or customizing internal systems according to the company's needs.

6. Backup & Recovery (Backup & Recovery)

Setting up the Patrolling Backup systems.

Testing data recovery plans after accidents or breakdowns.

The IT department is not just a “computer troubleshooting”, it is a strategic team that maintains security, efficiency, and business continuity. Without them, the institution stops!

Question 1: what is a floating fixed path

(Floating Static Route), and what is it used for?

Question: What is a Floating Static Route, and why is it used?

ANSWER: Is a fixed path consisting of an administrative distance (AD) higher than the main path, so that it is only used as a backup path in case the main path fails.

Answer: It's a static route configured with a higher administrative distance (AD) than the primary route, so it is only used as a backup path if the main route fails.

Question 2: What is the purpose of the VTP (VLAN Trunking Protocol)?

Question: What is the purpose of VTP (VLAN Trunking Protocol)?

Answer: VLANs databases synchronize between converters in the same domain, making it easier to manage VLANs and reduces errors.

Answer: It allows VLAN databases to be synchronized among switches within the same domain, which simplifies VLAN management and reduces errors.

Question 3: What are the default Subnet Masks for the following IPv4 title categories: A, B, and C?

Question: What are the default subnet masks for the following IPv4 address classes: A, B, and C?

The answer is :

Category A: 255.0.0.0

Category B: 255.255.0.0

Category C: 255.255.255.0

Answer:

Class A: 255.0.0.0

Class B: 255.255.0.0

Class C: 255.255.255.0

Question 4: What is the basic function of Wireless Access Point (WAP)?

Question: What is the primary function of a Wireless Access Point (WAP)?

Answer: Connecting wireless devices to a wired network, allowing them to access network and Internet resources.

Answer: It connects wireless devices to a wired network, allowing them to access network resources and the internet.

Q 5: What is the difference between Standard ACL and Extended ACL?

Question: What is the difference between a Standard ACL and an Extended ACL?

Answer: Standard ACL filters traffic based only on the source IP address, while Extended ACL allows filtering based on the source IP title, destination, port number, and protocol.

Answer: Standard ACLs filter traffic based only on the source IP address, while Extended ACLs allow filtering based on the source and destination IP addresses, port number, and protocol.

ingSniffing: How does it happen? What does he reveal? And how do we protect ourselves?

Moral and Legal Warning: Any use of this knowledge outside of your authorized/owned environment is against the law. Goal is my defense awareness

1) What is tapping?

Is to capture network traffic (Packets) to understand what the ends are exchanging. In modern networks (Switches) the attacker often needs to access the On-Path via methods such as:

ARP (converting traffic through device)

Weak or fake wifi access points

Internal resources

(Activating SPAN/Port-Mirror from a malicious employee)

Home devices on the host (Agent/Proxy malicious)

Bottom line: If the data isn't well encrypted end-to-end, the readable content can be seen or edited.

2) What can be revealed?

Encryptionless: URLs, Login Data

(Telnet/FTP/POP3/HTTP), messages content

With partial encryption: metadata (IP addresses, ports, call timing and frequency, host name visible in SNI, absence ECH, size of packages).

TLS 1.3 + HTTPS: The content is encrypted, but some metadata is still visible on the network (not on the device itself if affected).

3) A safe lab for learning (only on your network)

These are the defensive learning steps to understand what shows someone picking up your device's movement inside my home lab:

a) tcpdump (capturing the movement of your device)

Just capture the motion of your device to review what is being sent and how it looks when encrypted.

#Capture sessions only on your device (replace IP with your device address)

sudo tcpdump -i eth0 -n host 192.168.1.50 -w your_traffic.pcap

Open the Wireshark file and watch:

Useful display filters:

dns — DNS queries (is it encrypted? )

tls.handshake — TLS negotiation (releases and testimonies)

http.request — Must be semi-missing if you impose HTTPS-Only

dhcp — for those who see the distributed IP addresses

Check that sensitive sites are not showing Requests with clear text (HTTP), and that old protocols (FTP/Telnet/POP3) are not used.

b) Check the strength of encryption

At Wireshark Click TLS/QUIC session:

TLS 1.3 or HTTP/3 (QUIC) are preferred.

Watch the Testimony Series: The source (CA) must be known and appropriate—unexpected internal CA appearance (SSL Interception) is unauthorized.

Golden Fact: Having older protocols such as SSLv3/TLS 1.0 or algorithms without PFS indicates that sessions can be read later if the main key is detected.

4) How does tapping become possible despite the switch?

Summary of techniques to put the striker himself on the course:

ARP Spoofing/Poisoning: fakes IP .MAC tying to cross traffic.

MAC Flooding: Drowning CAM schedule on the switch, temporarily turns closer to Hub.

Evil Twin Wi-Fi: a fake access point under your network name attracts devices.

Local Proxy/Agent): The hazard—even TLS can be broken with a locally grown root certificate.

>These styles are detected and prevent settings on the switch/wifi and the host as in the following sections.

5) Risk Indicators (IOCs) indicating the presence of Sniffing/MITM

Sudden change in the ARP table on devices:

Same IP address linked to different MAC or vice versa.

Unexpected CA appearance in TLS certifications inside the browser.

Unrequired EDR/IDS Alerts on ARP Replies or Big Hikes in gratuitous ARP.

Increase of DNS inquiries towards an unauthorized analyst.

In wireless networks: Client devices connect to a higher signal strength but with the same SSID and a different unknown BSSID.

6) دفاع طبقي (Defense-in-Depth)

A) For Users/Home:

WPA3-SAE (or WPA2-PSK strong + WPS disable), and PMF/802.11w activation to protect management frameworks.

Long Wi-Fi password (12–16+ random), and guest network with Client Isolation.

Enable HTTPS-Only mode in the browser, and avoid HTTP.

encrypted DNS (DoH/DoT) or VPN on public networks.

Upgrading the router, disabling remotely, changing the password on the virtual control panel.

On phone/laptop: disconnect auto-connect to open networks, delete old networks.

(b) for the institutions:

802.1X/EAP-TLS to impose identification before accessing the network.

ميزات السويتش: DHCP Snooping, Dynamic ARP Inspection, IP Source Guard, Port Security.

Micro-segmentation and unnecessary broadcast reduction.

TLS 1.3, force HSTS on your services, support mTLS where needed.

Use EDR + NDR (Zeek/Suricata) to monitor network patterns, and NetFlow/IPFIX to analyze behavior.

Monitor TLS certifications issued for your field via CT reports, and detect any unsolved SSL Interception.

NAC to reject unmanaged/unupdated devices.

On the physical class: Lock switch ports, track SPAN/Port-Mirror changes via logs and minutes.

7) Rapid response in suspicion (Incident Mini-Playbook)

1. Isolate the suspected sector/port (VLAN Quarantine).

2. Cluster of Clues: PCAP Short from Credible Viewpoint + ARP Schedules from Multiple Devices + TLS Certificate Snapshots.

3. Check the Switch: DAI/DHCP Snooping/Port-Mirror Records—Who did what and when?

4. Change sensitive passwords (Wi-Fi, network devices, portals).

5. Lease Reset if there is a malfunction with DHCP/ARP.

6. Notify users to temporarily avoid open networks, and impose institutional VPNs.

7. Trained the team on MITM indicators, developed policies to close the gap that allowed for the incident.

Common misconceptions (Myths vs Facts)

"HTTPS hides everything"

Hides content but not entirely hides metadata (like a destination, and sometimes host name via SNI unless ECH is used).

"Alternate networks completely ban tapping"

Block it by default, but local ARP/Port-Mirror/Agent attacks get bypassed.

"VPN always solve the problem"

Protects against intermediate network tapping, but if the device is hacked, the enemy is inside the castle.

9) Quick checklist (for home and business)

[] Disable WPS, enable WPA3/PMF

[] Changing router passwords and virtual control panel

[] Periodically updating the firmware

[] Implementation of HTTPS-Only and modern TLS policies

[] enable DoH/DoT or use a VPN when needed

[] Applying 802.1X, DAI, DHCP Snooping and Port Security in work environments

[] monitoring TLS certificates and finding any unfamiliar CA

[] League training on social media awareness and Evil Twin

Tapping isn't magic—it's the absence of proper coding or an attacker's success in positioning himself on track. With layered defenses, modern encryption, and smart surveillance, the surface of attack is significantly reduced.

Day 36 – What is SSH in Networking?

Welcome to our Learning Networking Series! Having talked about security and access control protocols like RADIUS and TACACS+,

Today we are covering a basic protocol used to secure access to and manage devices remotely: the Secure Shell Protocol (SSH).

What is the SSH? (Secure Shell)

SSH is a crypto networking protocol used for secure access and remotely management of devices.

Simply, a very secure alternative to old, non-encrypted protocols like Telnet and rlogin.

Imagine wanting to control a router in another office or even another continent.

* If you use an old protocol like Telnet, everything you write (including username and password) is sent as a plain, non-encrypted text.

Anyone watching the network can easily see this data and steal it.

* As for SSH, all data that is sent and received between you and the device is fully encrypted.

This ensures that your conversation with the device is confidential and no one can object or understand.

- The SSH is a protocol used for the secure access to devices remotely, and it encrypts all data to ensure confidentiality and integrity.

Why is SSH essential in modern networks?

The importance of SSH comes from its ability to provide a safe and reliable working environment for infrastructure management:

*Strong encryption: SSH uses advanced encryption algorithms to protect data, preventing sensitive information like passwords from tapping or theft.

* Secure Authentication: SSH supports robust authentication methods, highlights:

*Password Authentication: Like any normal login system, but it sends password encrypted.

*Public Key Authentication: considered safer than passwords. User create a pair of keys

(private key and public key)

The public key is placed on the device to be accessed, while the user keeps the private key a secret. To access, no password is needed, two keys are checked.

*Flexibility: SSH is not limited to device management.

Can also be used to secure file transfers

(Using SFTP or SCP protocol) or even to create encrypted channels (SSH Tunneling) to send other types of traffic.

* Integrity: SSH ensures that the data being sent has not been modified during transit.

- SSH is essential to secure remote management, it protects your data through strong encryption and authentication, and supports many other secure applications.

How does the SSH work? (A process in simple steps)

Contacting SSH takes specific steps to ensure security:

*Negotiation: When the user starts calling, the client and the server negotiate the protocol version and the best encryption algorithms to be used.

* Authentication: After agreement, the server starts the authentication process to verify the user's identity.

This process can be via password or password.

*Channel Establishment): Once authentication is successful, a secure and encrypted communication channel between client and server is established.

* Data Transfer: All data that is written by the user in the command line or sent by the device starts in transfer via this encrypted channel.

What about the old Telnet protocol?

Telnet is a protocol similar to SSH but not completely encrypted.

This means it's user-friendly, but it's a great security threat in any modern networking environment.

So, it's been completely replaced by SSH.

SSH is not just a tool for managing devices, it is an essential principle of network security.

Ignoring the use of SSH and replacing it with unsafe protocols is like opening your door to everyone.

What SSH command do you use most in your business? Have you ever tried authenticating with a password instead of a password? Share with us in the comments!

*Question 1: What is the purpose of ip default-gateway?

Answer: It is used to assign a Default Gateway to a device that acts as Layer 2 (such as an adapter) so that it can access devices outside of its local network.

Answer: It is used to set the default gateway for a Layer 2 device (like a switch) so that it can reach devices outside its local network.

* Question 2: In the sub-network, what is the difference between the Network Address and the Host Address?

Question: In a subnet, what is the difference between a Network Address and a Host Address?

ANSWER: A network address is the first address in a domain and is used to define the network itself, while a host address is any other address in a domain that can be assigned to an individual device.

Answer: The network address is the first address in the range and is used to identify the network itself, while a host address is any other address in the range that can be assigned to an individual device.

* Question 3: In the OSI model, which tier is responsible for Flow Control and Error Checking?

Question: In the OSI model, which layer is responsible for Flow Control and Error Checking?

Answer: fourth layer (Transport Layer)

Answer: Layer 4 (Transport Layer).

*Question 4: what are the two major types of dynamic NAT?

Question: What are the two main types of dynamic NAT?

Answer: Dynamic NAT (using a set of general addresses) and PAT (Port Address Translation) that uses one general address.

Answer: Dynamic NAT (which uses a pool of public addresses) and PAT (Port Address Translation), which uses a single public address.

*Question 5: How does a converter deal with a frame with an unknown destination MAC title?

Question: How does a switch handle a frame with an unknown destination MAC address?

Answer: Sends the frame to all ports except the one it received from (Flooding), then the MAC address learns the destination when the target device returns.

Answer: It floods the frame out of all ports except the one it was received on, and then learns the destination MAC address when the target device replies.

Day 36: What is TACACS+ in Networking? | Day 36 –

What is TACACS+ in Networking?

Welcome back to our Learning Networking series! After we talked about the RADIUS protocol, today we're going to talk about another very important network security protocol, which is often compared to RADIUS: Access System for Devices and Communications (TACACS+).

What is TACACS+?

(Terminal Access Controller Access-Control System Plus)

TACACS+ is a centralized protocol, developed by Cisco, used to manage Authentication, Authorization, Accounting, or AAA services.

Although its primary function is similar to RADIUS, there are fundamental differences that make it a preferred choice in certain environments, especially in device management.

While RADIUS focuses more on authorizing users to access the network (like connecting to Wi-Fi),

TACACS+ focuses more on authenticating and imagining network executives to access the network devices themselves

(Routers, adapters, fire walls) and managing their settings.

- TACACS+ is a centralized AAA protocol that is primarily used to control network administrators' access to devices and determine their capabilities.

The main differences between TACACS+ and RADIUS

Understanding these differences is key to choosing the right protocol for your work environment:

1. The Protocol :

TACACS+: Using TCP protocol, exit 49.

RADIUS: uses UDP protocol, Executives 1812 (for authentication and visualization) and 1813 (for accounting).

2. Encryption:

TACACS+: Encrypts the entire data package, providing high security.

RADIUS: Only the password is encrypted, while the rest of the data (such as usernames) is non-encrypted.

3. AAA Chapter of Service:

TACACS+: Differentiates Authentication, Authorization and Accounting services. Different protocols and servers can be used for each service.

RADIUS: combines authentication and imagination services in one package.

4. Authorization (Authorization):

TACACS+: Allows for detailed Per-command Authorization. Commands can be set that each user is allowed to do on each device.

RADIUS: provides a limited imagination, often limited to general acceptance or denial of access.

5. Basic use of :

TACACS+: Specifically designed to control access of network officials to the network devices itself (routers, adapters, walls).

RADIUS: Designed to control end users access to the network (such as wireless network or VPN).

6. RELIABILITY :

TACACS+: More reliable because of its use of TCP protocol that ensures data access.

RADIUS: Faster but less reliable because of its use of UDP protocol which does not guarantee data access.

While RADIUS is better for user authentication, TACACS+ excels in device management thanks to robust encryption, AAA service separation, and precise command control.

How does TACACS+ work? ( Work flow )

The TACACS+ course is similar to RADIUS but more detailed, especially in the imaging stage:

* Authentication request :

* Admin is trying to log in to a router, for example.

* The router (TACACS+ client) sends username and password to the TACACS+ server.

* The server verifies the user's identity in his base and sends a response with accept or decline.

*Authorization request:

* After successful authentication, the device sends another conversion request to the TACACS+ server, specifying what the administrator is trying to execute (e.g., configure terminal).

*Server checks if this admin has the authority to execute this matter.

* The server sends a response by the device, either by accepting the execution or rejecting it.

*Accounting request :

*TACACS+ server records all orders executed, when they were executed, and who executed them.

* This information is critical for security review and compliance.

The TACACS+ course features a detailed conversion step for each command, giving network executives unparalleled control over access capabilities.

TACACS+ is the ideal choice for any organization with strict security requirements, especially when it comes to network infrastructure management.

It provides an extra layer of security to ensure that only authorized persons have access to devices, have specific powers and fully monitored.

Do you work in an environment that uses TACACS+ to manage network devices? What is the most important feature that you consider to be necessary to control devices? Share with us in the comments!

Day 35: What is the RADIUS in Networks? | Day 35 –

What is RADIUS in Networking?

Welcome to day 35 of our Learn to Network series! Having covered many of the protocols and techniques that operate networks, today we'll dive into a crucial security protocol that is mostly used in wired and wireless networks:

Remote authentication service via mobile user (RADIUS)

What is a RADIUS?

(Remote Authentication Dial-In User Service)

The RADIUS is a central network protocol used for presentation

Authentication and intrusion services

Authorization and Accounting, also known as AAA for short.

Imagine you're in a big corporation, and whenever you try to call the network (whether wire or wireless) you're asked for a username and password. The RADIUS is the protocol that stands behind the scenes to manage this whole process.

Instead of checking every network device

(Wireless access point, adapter) by itself from the user's identity, it sends the request to a central RADIUS server.

This server acts as a network “security guard”, which has a database of all users allowed to access and permissions.

The RADIUS is a central protocol used to manage Authentication, Imagination and Accounting (AAA) services in networks, ensuring that only authorized users have access to the network resources.

Basic Components and RADIUS Protocol Workshop

RADIUS system consists of three main components that work together to ensure safety:

*Client/User): is a person or device attempting to access the network (such as a computer, smartphone, etc).

* عميل RADIUS (RADIUS Client):

* Is a network device that receives a call request from the user, such as a wireless Access Point (Access Point) or an adapter (Switch).

* This device is not the one who authenticates itself, but passes the authentication request from the user to the RADIUS server.

*also referred to as a name

NAS (Network Access Server).

* خادم RADIUS (RADIUS Server):

* Is a central server that has a user database, passwords, and permissions.

* Receives verification requests from RADIUS customers, verifies user identity, then sends a response with acceptance or rejection.

Workflow (example on a wireless network):

* The user tries to connect with a wireless Access Point.

* Send the access point (RADIUS client) username and password to the RADIUS server.

* RADIUS server checks user's data in its base.

* If the data is correct, the RADIUS server sends an Access-Accept response to the access point, and this response may include additional information about the user's permissions (e.g., the VLAN in which the user will be placed).

* Access point allows user to connect to the network.

* If the data is incorrect, the RADIUS server sends an Access-Reject response and the user is blocked from contacting.

RADIUS course consists of three ends:

The user requesting access, the RADIUS client (like access point) that passes the request, and the RADIUS central server that does authentication and visualization.

RADIUS features and the reason for its superiority

*Centralization): Instead of storing user data on each individual network device, it is stored in one central location (RADIUS server), facilitating management and updates.

*Security: Provides a strong layer of security by verifying the identity of each user before allowing them to access the network. It also encrypts password in verification messages to increase protection.

*Authorization): Its role is not limited to acceptance or rejection, but it can determine the user's permissions after authorization (e.g. granting access to a certain network, or imposing certain restrictions on its use of the network).

* Accounting: The RADIUS server can record detailed information about the user's network usage, such as contact time, amount of data transferred, and time spent online. This information is crucial for billing or monitoring purposes.

*Flexibility: RADIUS is used in a variety of environments, including Wireless (802.1x), Dial-in Mobile Access Networks, and VPNs.

RADIUS provides a centralized, secure, and flexible solution for network access management, making it the essential AAA Protocol in the majority of institutions.

The RADIUS is the cornerstone of modern network security.

Understanding how it works and how it is set up is an essential skill for any network security and infrastructure management specialist.

Have you ever encountered a network login screen that requires a username and password? Probably RADIUS was the one checking your identity! Share your experience or any question you have about this protocol in the comments!

Day 34: What is NetFlow in Networking? | Day 34 – What is NetFlow in Networking?

Welcome back to our Learning Networking series! After we've talked about complex routing techniques, today we're going to tackle a powerful and very important tool in the field of monitoring and analysis of network movement: NetFlow.

What is NetFlow?

NetFlow is a technology developed by Cisco to collect information about IP traffic flow that passes through a network device. Imagine your network is a city, and traffic is cars.

Traditional steering and adapters only tell you "how many cars" are in or out of town.

As for NetFlow, it gives you much finer details:

*where did all the cars come from? (Source IP address)

* Where did you go? ( Destination IP address)

*What type of load was she carrying? (Portrait and Protocol Number)

* How many cars per trip? ( Number of Packs )

* How long is the trip? (Time to start and end the stream)

Basically, NetFlow collects information about every “flow” on the network, and flow is known as a series of packages that share the same criteria (such as source, destination, protocol).

NetFlow is a technology used to collect and analyze IP network traffic flow data, providing an in-depth view of what is happening on the network.

Why do we use NetFlow for network management?

NetFlow's data is a real treasure for network executives, and it's used for multiple purposes:

*Performance Monitoring and Analysis: Allows to understand how to use the network, who are the most used frequency band users, and which applications are the most used. This helps with capacity planning and optimizing performance.

* Security and Fault Detection: NetFlow data is used to detect suspicious behavior or attacks.

For example, it can detect a DDoS (DDoS) attack by identifying massive flows from multiple sources to a single destination.

It also helps to identify the source of problems when there is slowness in the network.

* Accounting: Internet service providers (ISPs) can use NetFlow to calculate customer use for data, which helps with billing.

* Traffic Analysis: Provides valuable insights into traffic patterns, which helps improve network design and orientation policies.

NetFlow is essential for monitoring network performance, enhancing security, detecting errors, and understanding traffic patterns.

How does NetFlow work? ( The basic ingredients )

NetFlow system consists of three main parts:

* Source (NetFlow Exporter):

* Is a router or converter (that supports NetFlow) that collects flow information from data packages that passes through it.

*Creates a Flow Record for each new flow, then releases (sends) these records to the NetFlow Collector.

*This is the basic source of data.

* The Collector (NetFlow Collector):

* Is a server or central application that receives flow logs from all devices on the network.

*stores and processes this raw data.

*It can be a specialized software such as ManageEngine NetFlow Analyzer or SolarWinds NetFlow Traffic Analyzer.

* The Analyzer (NetFlow Analyzer):

* Is a user interface or program that allows the network administrator to view and analyze the data collected by the "collective".

*Provides reports and diagrams that show who is talking to who, what apps are most used, and how much data is transmitted.

The operation in a nutshell:

A network device (sourcer) collects information about streams and sends it to a central server (the collector), which in turn analyzes it and displays it in a graphical interface (analyzer) to facilitate understanding.

NetFlow is more than just a monitoring tool; it gives network managers a complete and comprehensive "view" of what is happening within their networks, enabling them to make informed decisions about performance and security.

Have you ever used a tool to monitor traffic on your network? What is the most important feature for you in network motion analysis? Share with us in the comments!

Question 1: What is the Administrative Distance for the OSPF protocol?

Question: What is the administrative distance of OSPF?

The answer: 110.

Answer: 110.

* Question 2: What is the main difference between Straight-through cable and Crossover cable?

Question: What is the main difference between a Straight-through cable and a Crossover cable?

Answer: Straight-through cable is used to connect different devices (e.g. a PC to an adapter), while Crossover cable is used to tie similar devices (e. g. adapter to adapter or a PC to the PC).

Answer: A Straight-through cable is used to connect dissimilar devices (like a PC to a switch), while a Crossover cable is used to connect similar devices (like a switch to a switch or a PC to a PC).

*Q3: What is the purpose of the ip route on the Cisco router?

Question: What is the purpose of the ip route command on a Cisco router?

ANSWER: Used to create a static route, which directs traffic to a manually specified destination network.

Answer: It is used to create a static route, which manually directs traffic to a specific destination network.

*Question4: What is the primary function of Layer 2) Bridge OSI model?

Question: What is the primary function of a Bridge at Layer 2 of the OSI model?

Answer: It connects two different local networks (LANs), and filters traffic based on MAC addresses.

Answer: It connects two different LANs and filters traffic based on MAC addresses.

*Q5: What is the main difference between the show ip interface brief and the show interfaces?

Question: What is the main difference between the show ip interface brief and show interfaces commands?

Answer: show ip interface brief provides a brief summary of the condition of the interfaces, while show interfaces displays detailed, complete statistics for each interface.

Answer: show ip interface brief provides a brief summary of interface status, while show interfaces displays detailed and complete statistics for each interface.